### Logic Explanation

- Problem Setup
 The code simulates 1D nonlinear heat conduction in a rod, where thermal conductivity depends on temperature:

- The goal is to compute the temperature u(x,t)u(x,t)u(x,t) over space and time for different nonlinearity parameters χ\chiχ.

- Spatial and Temporal Discretization

- Divide the rod into N points (x grid).

- Divide the total time into M steps (t grid).

- Use Implicit Euler method for time discretization:

- which is unconditionally stable for nonlinear problems.

- Initial and Boundary Conditions

- Initial temperature along the rod is set according to a quadratic function (from the paper).

- Dirichlet boundary conditions: u(a,t) = alpha, u(b,t) = beta.

- Use Newton-Raphson method to iteratively solve for the temperature vector u at the new time step:

- Nonlinear FDM equations lead to a system of equations at each time step.

- Use Newton-Raphson method to iteratively solve for the temperature vector u at the new time step:

- Compute the residual vector G representing the difference from the FDM equations.

- Construct the Jacobian matrix L for the Newton update.

- Update the solution:

- Stop when the residual norm (eps) is below tolerance tol or maximum iterations maxIt is reached.

- Temperature-Dependent Conductivity

- For each interior point, compute:

  - k = kappa0 * exp(chi*u)

  - Dk = chi*k, D2k = chi*Dk

- These values modify the residual and Jacobian, capturing the nonlinearity.

- Time Stepping Loop

- Start with initial temperature u0.

- For each time step:

  - Initialize Newton guess with the previous step solution.

  - Run Newton iterations until convergence.

  - Store the converged solution in U(:,n).

- Plotting

- 3D mesh plot: temperature vs. space and time.

- 2D plot: final temperature distribution along the rod.

- Repeat for each chi value to visualize the effect of nonlinearity.

In short:

- Discretize space and time → Apply Implicit Euler → Solve nonlinear system with Newton → Store and plot results.

- This approach ensures stability and accuracy even for strongly nonlinear thermal conductivity.